CS23532-COMPUTER NETWORKS-LAB MANUAL

PRACTICAL – 12

AIM:

To implement (a) an echo client-server using TCP and UDP sockets and (b) a chat program using socket programming, for end-to-end communication at the transport layer.

ALGORITHM / PROCEDURE:

(a1) TCP Echo Server and Client

- Server: Create a TCP socket using socket(AF_INET, SOCK_STREAM), bind it to an IP address and port 5000, call listen() and wait in accept() for the client.

- Client: Create a TCP socket and call connect() with the IP address and port of the server.

- The client sends a message using send(). The server reads it with recv(), displays it and sends the same data back using sendall().

- The client receives the echoed message and displays it. This is repeated for each message.

- Close the sockets using close().

(a2) UDP Echo Server and Client

- Server: Create a UDP socket with SOCK_DGRAM and bind it to port 6000. No listen() or accept() is needed because UDP is connectionless.

- Client: Create a UDP socket and send the message to the server address using sendto().

- The server receives the datagram and client address using recvfrom() and sends it back using sendto().

- The client receives the echo using recvfrom() and displays it. Close the sockets.

(b) Chat Program

- The chat server creates a TCP socket on port 7000 and waits for a client. The client connects to it.

- The client types a message and sends it. The server displays it, then the server user types the reply and sends it back.

- Messages are exchanged alternately until either side sends the word exit. Then both sockets are closed.

| Point | TCP | UDP |
| --- | --- | --- |
| Socket type | SOCK_STREAM | SOCK_DGRAM |
| Connection | Connection oriented (3-way handshake) | Connectionless |
| Reliability | Reliable, ordered delivery | No guarantee |
| Server calls | bind, listen, accept, recv, send | bind, recvfrom, sendto |
| Used in | HTTP, FTP, SMTP | DNS, DHCP, streaming |

# DIAGRAM:

TCP Server	TCP Client

socket()

close()

data

recv()

connect()

send()

listen()

socket()

bind()

recv()

accept()

echo of data

send()

close()

connection request (3-way handshake)

Fig 1: Socket calls in TCP client-server communication

UDP Server	UDP Client

bind()

recvfrom()

datagram

sendto()

echo of datagram

socket()

close()

recvfrom()

sendto()

close()

socket()

Fig 2: Socket calls in UDP client-server communication

PROGRAM CODE:

- echo_tcp_server.py:

import socket

s = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

s.bind(("127.0.0.1", 5000))

s.listen(1)

print("TCP Echo Server listening on port 5000 ...")

conn, addr = s.accept()

print("Connected by", addr)

while True:

data = conn.recv(1024)

if not data:

break

print("Received :", data.decode())

conn.sendall(data)

conn.close()

s.close()

print("Connection closed.")

- echo_tcp_client.py:

import socket

c = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

c.connect(("127.0.0.1", 5000))

for msg in ["Hello Server", "Computer Networks Lab", "bye"]:

c.sendall(msg.encode())

print("Sent     :", msg)

print("Echoed   :", c.recv(1024).decode())

c.close()

- echo_udp_server.py:

import socket

s = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

s.bind(("127.0.0.1", 6000))

print("UDP Echo Server waiting on port 6000 ...")

for _ in range(2):

data, addr = s.recvfrom(1024)

print("Received from", addr, ":", data.decode())

s.sendto(data, addr)

s.close()

- echo_udp_client.py:

import socket

c = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

for msg in ["Hello UDP", "Datagram test"]:

c.sendto(msg.encode(), ("127.0.0.1", 6000))

data, _ = c.recvfrom(1024)

print("Sent     :", msg)

print("Echoed   :", data.decode())

c.close()

- chat_server.py:

import socket

s = socket.socket()

s.bind(("127.0.0.1", 7000))

s.listen(1)

print("Chat server started. Waiting for client ...")

conn, addr = s.accept()

print("Client connected:", addr)

while True:

msg = conn.recv(1024).decode()

if not msg or msg.lower() == "exit":

break

print("Client :", msg)

reply = input("Server : ")

conn.send(reply.encode())

if reply.lower() == "exit":

break

conn.close()

s.close()

print("Chat ended.")

- chat_client.py:

import socket

c = socket.socket()

c.connect(("127.0.0.1", 7000))

print("Connected to chat server. Type 'exit' to quit.")

while True:

msg = input("Client : ")

c.send(msg.encode())

if msg.lower() == "exit":

break

reply = c.recv(1024).decode()

if reply.lower() == "exit":

break

print("Server :", reply)

c.close()

print("Chat ended.")

OUTPUT:

(a1) TCP Echo:

Server output:

TCP Echo Server listening on port 5000 ...

Connected by ('127.0.0.1', 46248)

Received : Hello Server

Received : Computer Networks Lab

Received : bye

Connection closed.

Client ouput:

Sent     : Hello Server

Echoed   : Hello Server

Sent     : Computer Networks Lab

Echoed   : Computer Networks Lab

Sent     : bye

Echoed   : bye

(a2) UDP Echo:

Server output:

UDP Echo Server waiting on port 6000 ...

Received from ('127.0.0.1', 53205) : Hello UDP

Received from ('127.0.0.1', 53205) : Datagram test

Client output:

Sent     : Hello UDP

Echoed   : Hello UDP

Sent     : Datagram test

Echoed   : Datagram test

(b) Chat Program:

Server output:

Chat server started. Waiting for client ...

Client connected: ('127.0.0.1', 43260)

Client : Hi Server, this is User

Server : Hello User, how can I help?

Client : Is the CN lab record ready?

Server : Yes, the lab record is due on Friday.

Client : Thanks, bye

Server : exit

Chat ended.

Client output:

Connected to chat server. Type 'exit' to quit.

Client : Hi Server, this is User

Server : Hello User, how can I help?

Client : Is the CN lab record ready?

Server : Yes, the lab record is due on Friday.

Client : Thanks, bye

Chat ended.

RESULT:

Thus, the echo client-server programs using TCP and UDP sockets and the chat program using socket programming were implemented and executed successfully.