CS23532-COMPUTER NETWORKS-LAB MANUAL

PRACTICAL - 8

AIM:

To use Nmap (Network Mapper) on the TryHackMe platform to discover live hosts in a network using different host-discovery techniques such as ARP, ICMP, TCP, and UDP scans, and to analyze the scan results.

PROCEDURE:

Step 1: Open TryHackMe

- Log in to the TryHackMe platform.

- Open the relevant Nmap/network-scanning room.

- Start the target machine.

- Start the AttackBox or connect to the TryHackMe network through VPN.

- Open the terminal.

### Step 2: Check Nmap installation

### Execute:  nmap –version

### Step 3: Identify the target

### Use the target IP address provided by the TryHackMe machine.

### Step 4: ARP Host Discovery

### ARP discovery is mainly useful when scanning hosts on the same local Ethernet network.

### Commands:

- sudo nmap -sn -PR 10.48.163.95

- For a subnet: sudo nmap -sn -PR 10.48.163.0/24

### Step 5: ICMP Host Discovery

### ICMP Echo Requests can be used to determine whether a host is reachable.

### Commands:

- sudo nmap -sn -PE 10.48.163.95

- sudo nmap -sn -PE 10.48.163.0/24

### Result:

### Host is up

### Step 6: TCP Host Discovery

### TCP-based discovery can be used when ICMP requests are blocked.

### Commands:

- TCP SYN Discovery: sudo nmap -sn -PS80,443 10.48.163.95

### Result:

### Host is up

### Step 7: TCP ACK Discovery

### Another TCP-based discovery method uses TCP ACK packets.

### Commands:

- sudo nmap -sn -PA80,443 10.48.163.95

### Result:

### Host is up

### Step 8: UDP Host Discovery

### UDP probes can also be used for host discovery.

### Commands:

- sudo nmap -sn -PU53,161 10.48.163.95

### Result:

### Host is up.

### Nmap done: 1 IP address (1 host up) scanned in 0.12 seconds

### Step 9: Combined Host Discovery

### Nmap can also use multiple discovery techniques.

### Commands:

- sudo nmap -sn -PE -PS80,443 -PU53 10.48.163.95

### This performs host discovery using:

- ICMP Echo

- TCP SYN

- UDP probes

### Result:

### Host is up (0.10s latency).

### Nmap done: 1 IP address (1 host up) scanned in 0.12 seconds

### Step 10: Basic Nmap Scan for Further Verification

### A normal Nmap scan can be performed to identify open TCP ports.

### Commands:

- nmap 10.48.163.95

### Result:

### Host is up (0.10s latency).

### PORT     STATE SERVICE

### 22/tcp     open  ssh

### 80/tcp     open  http

### 443/tcp   open  https

### Nmap done: 1 IP address (1 host up) scanned in 2.15 seconds

### Result:

### Thus, Nmap was successfully used on the TryHackMe platform to discover live hosts using ARP, ICMP, TCP, and UDP host-discovery techniques. The scan results were analyzed to determine the availability of the target hosts and, where applicable, the open ports and services running on them.