CS23532-COMPUTER NETWORKS-LAB MANUAL

PRACTICAL – 9

AIM:

To design and implement multiple subnets in Cisco Packet Tracer, assign static IP addresses to devices, connect the subnets using a router, and simulate packet transmission to observe communication with and without a router.

PROCEDURE:

- Create the Network Topology:

  - Open Cisco Packet Tracer.

  - Place the following devices:

  - 1 Router — Cisco 2911

  - 2 Switches — Cisco 2960

  - 4 PCs

  - Connect the devices

- Create the subnets:

The network 192.168.10.0/24 was divided into two /25 subnets.

Subnet 1

- Network Address: 192.168.10.0

- Subnet Mask: 255.255.255.128

- Usable IP Range: 192.168.10.1 – 192.168.10.126

- Broadcast Address: 192.168.10.127

Subnet 2

- Network Address: 192.168.10.128

- Subnet Mask: 255.255.255.128

- Usable IP Range: 192.168.10.129 – 192.168.10.254

- Broadcast Address: 192.168.10.255

- Assign Static IP Addresses:

## Configure the PCs through:

## PC → Desktop → IP Configuration

- Configure the Router:

Open:

Router0 → CLI

Command prompt:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.10.1 255.255.255.128

no shutdown

exit

interface gigabitEthernet 0/1

ip address 192.168.10.129 255.255.255.128

no shutdown

exit

end

show ip interface brief

Outcome:

GigabitEthernet0/0    192.168.10.1      up    up

GigabitEthernet0/1    192.168.10.129    up    up

- Test Communication Between Different Subnets:

Open:

PC0 → Desktop → Command Prompt

Command:

ping 192.168.10.130

Outcome:

Packets: Sent = 4

Received = 4

Lost = 0

- Test Communication Within Subnet 2:

Open:

PC2 → Desktop → Command Prompt

Command:

ping 192.168.10.140

Outcome:

Packets: Sent = 4

Received = 4

Lost = 0

- Simulate Packet Transmission:

- Switch from Realtime mode to Simulation mode.

- Select Add Simple PDU.

- Click PC0 and then PC2.

- Click Auto Capture / Play.

- Observe the packet travelling through:

- PC0 → Switch0 → Router0 → Switch1 → PC2

- Test Communication Without the Router:

- Return to Realtime mode.

- Disconnect the cable between Switch0 and Router0.

- Do not delete the router or switches.

- From PC0 → Desktop → Command Prompt : ping 192.168.10.130

Outcome:

Packets: Sent = 4

Received = 0

Lost = 4

SCREENSHOTS:

RESULT:

Thus, the experiment successfully demonstrated subnetting, static IP addressing, router-based inter-subnet communication, and the importance of a router for communication between different subnets.